2007 Mazdaspeed3 ECU Connector Pinout

Contributed by rotus8
Connector A

[image: image1.png]@..9]
FUEL INUECTOHR

ovim =g

MVIAI%WIHI

i oeM «-i:f |

WwBE BUOX COVEN

e

IGNITION BWITOH(ICGHT)
(SECTION 0140 9a)

c6

RELAY MELAY
{INJECTOR} : (ETV) 2
i l MAIN FUSE BLOCK
' G(EM
G/Y (F) B/Y(F) z(: )
no 0140-302
i REAR HO2S
| TR [G740.305
G/Y (EM)
G/L(EM) , ‘
B/Y(EM) ‘
IGNITION SWITCH(IG1) ‘ B/R(EM) W(EM) BR(EM)
(SECTION 0140-3a) . R(EM) R(EM)
; L .. R[0140-301B ~ M 0140-301BJA L 0140-301A]
o @ © © @ &S BC
PCM SENSOR GROUND
777
0140-301A 0140-301B 0140-302
PCM PCM REAR HO2S
1BE 1BA 1AW  1AS 1AD 1AK 1AG 1AC 1y |1s) 1Q iM " 1€ 1A e YBA AW PAS 2AQ  PAK 2AG 2AC 2Y U 20 M 2 2E 2A
18F 1BB 1AX 1AT 1AP 1AL 1AH 1A0 17 1wV 1R 1N J 1+ 18 2HE PAT  2AP  2AL 2AH 2AD 27 2V 2R 2N ) 2F 2B

1BC
RN

1BG
1BH

1AY
1AZ

1AL 1AQ
1AV 1AR

1AM 1Al
1AN  1AJ

1AE
1AF

1AA
1AR

PRH  PAX

28G
e

2BC
26D

PAY
242

SAU 2AQ 2AM
28V 2AIRN ZAN

2al
PAL

2AE

2AF  2AB 2X

PAA OW

26

Fan

2Q
ap

Refer to 00F section, 00J section and 00C section for FUSE BOX, PASSENGER JUNCTION BOX (PJB) and COMMON CONNECTOR LIST.

CONTROL SYSTEM (L3 WITH TC)

0140-3b





	Pin#
	Color
	Function

	1A
	Unused
	Unused

	1B
	Black/Red
	Starter Interlock Switch

	1C
	Unused
	Unused

	1D
	Black/Yellow
	Clutch Pedal Position Switch

	1E
	Unused
	Unused

	1F
	Unused
	Unused

	1G
	Unused
	Unused

	1H
	Black/Blue
	Fuel Pump Relay

	1I
	Black/Yellow
	Air Conditioning relay

	1J
	Black/Green
	Refrigerant Pressure Switch

	1K
	Unused
	Unused

	1L
	Unused
	Unused

	1M
	White/Blue
	MAF sensor/Intake Air Temperature sensor

	1N
	Unused
	Unused

	1O
	Unused
	Unused

	1P
	Black
	MAF sensor/Intake Air Temperature sensor

	1Q
	Unused
	Unused

	1R
	Unused
	Unused

	1S
	Black/Orange
	Neutral Switch

	1T
	Unused
	Unused

	1U
	Black
	Accelerator Pedal Position sensor

	1V
	Brown/White
	Evap System Leak Detection Pump coil

	1W
	Unused
	Unused

	1X
	Unused
	Unused

	1Y
	Grey
	Accelerator Pedal Position sensor

	1Z
	Unused
	Unused

	1AA
	Black/White
	Fuel Pump Speed Control Relay

	1AB
	Red/Blue
	Brake Switch

	1AC
	White
	Accelerator Pedal Position sensor

	1AD
	Unused
	Unused

	1AE
	Blue
	Fan Control Module

	1AF
	Unused
	Unused

	1AG
	Unused
	Unused

	1AH
	Unused
	Unused

	1AI
	Blue/Red
	HS CAN_L Data Link Connector

	1AJ
	Green/Black
	Accelerator Pedal Position sensor

	1AK
	White/Red
	MAF sensor/Intake Air Temperature sensor

	1AL
	Unused
	Unused

	1AM
	Grey/Red
	HS CAN_H Data Link Connector

	1AN
	Unused
	Unused

	1AO
	Unused
	Unused

	1AP
	Unused
	Unused

	1AQ
	White
	Cruise Control Switch

	1AR
	Brown/Blue
	MAF sensor/Intake Air Temperature sensor

	1AS
	White/Violet
	Evap System Leak Detection Pump motor

	1AT
	Black/Blue
	Main relay control

	1AU
	Pink/Black
	Refrigerant Pressure Switch

	1AV
	Yellow
	Accelerator Pedal Position sensor

	1AW
	Green/Blue
	Fuel Injector Relay control

	1AX
	Black/Yellow
	Drive By Wire Relay control

	1AY
	Green/Blue
	+12V from ignition switch, PJB F69, 20A

	1AZ
	Black
	Ground

	1BA
	Red
	+12V from fuse F30, 10A

	1BB
	Black
	Ground

	1BC
	Brown
	Rear Oxygen Sensor ground

	1BD
	Black
	Ground

	1BE
	Green/Orange
	Main relay out fuse F36, 10A

	1BF
	Green/Yellow
	Power from Drive By Wire Relay

	1BG
	Black
	Ground

	1BH
	Black
	Ground


Connector B
	Pin#
	Color
	Function

	2A
	Green/Red
	Throttle Body motor

	2B
	Black/Orange
	Throttle Body motor

	2C
	Brown/Red
	Front O2 sensor

	2D
	Black/Red
	Rear Oxygen Sensor Heater

	2E
	Green/Blue
	Main relay out fuse F36, 10A

	2F
	Black/Orange
	High Pressure Fuel Pump

	2G
	Yellow
	High Pressure Fuel Pump

	2H
	Black
	Ground

	2I
	Green/Black
	Fuel Pressure sensor reference voltage

	2J
	Unused
	Unused

	2K
	Unused
	Unused

	2L
	Unused
	Unused

	2M
	Black/Blue
	Front O2 sensor

	2N
	Brown/Green
	MAP sensor/Boost Air Temp sensor

	2O
	Unused
	Unused

	2P
	Brown/Green
	Crank Position sensor ground

	2Q
	White
	Rear Oxygen sensor signal

	2R
	White/Green
	Fuel Pressure sensor

	2S
	Yellow/Violet
	Camshaft Position sensor

	2T
	Yellow
	Power Steering Pressure Switch

	2U
	Brown/Yellow
	Knock sensor

	2V
	White/Black
	Knock sensor

	2W
	White/Red
	Crank Position sensor

	2X
	Black/Yellow
	Shield for wires to Front O2 sensor, Throttle Body, Crank Position sensor, Knock sensor, and Cam position sensor

	2Y
	Yellow/Red
	Front O2 sensor

	2Z
	White/Red
	Front O2 sensor

	2AA
	Black/White
	Wastegate Control Solenoid Valve

	2AB
	Black/Orange
	Purge Solenoid Valve

	2AC
	Grey/Red
	Front O2 sensor

	2AD
	Brown/Red
	Front O2 sensor

	2AE
	White/Black
	Variable Swirl Shutter Valve Switch

	2AF
	Black/Orange
	Oil Control Valve (Variable Valve Timing)

	2AG
	White/Green
	MAP sensor/Boost Air Temp sensor

	2AH
	White
	Engine Coolant Temperature sensor

	2AI
	Blue
	Generator

	2AJ
	Grey
	Generator

	2AK
	White/Blue
	Throttle Body position sensor

	2AL
	Yellow/Blue
	Throttle Body position sensor

	2AM
	Black/Red
	EGR Valve

	2AN
	Black/Green
	EGR Valve

	2AO
	Brown/Blue
	Throttle Body position sensor

	2AP
	White/Blue
	Throttle Body position sensor

	2AQ
	Black/White
	EGR Valve

	2AR
	Black/Orange
	EGR Valve

	2AS
	Black/Red
	Variable Swirl Solenoid Valve

	2AT
	White/Green
	Ignition Coil #4

	2AU
	Green/Black
	MAP sensor/Boost Air Temp sensor

	2AV
	Brown/Green
	MAP sensor/Boost Air Temp sensor

	2AW
	White/Red
	Ignition Coil #2

	2AX
	White/Blue
	Ignition Coil #3

	2AY
	Brown
	Engine Coolant Temperature sensor ground

	2AZ
	White/Blue
	Fuel Injector #4

	2BA
	White
	Ignition Coil #1

	2BB
	White/Violet
	Fuel Injector #1

	2BC
	White
	Fuel Injector #2

	2BD
	White/Red
	Fuel Injector #3

	2BE
	Red
	Power from Fuel Injector Relay

	2BF
	Red
	Power from Fuel Injector Relay

	2BG
	Yellow/Violet
	Fuel Injector #1 & #4 Common

	2BH
	Yellow/Red
	Fuel Injector #2 & #3 Common


