Mazda5 LED Panel Install
1) Remove original panels (behind bumper).
2) Mount LED Panels.
3) Locations.
- Light with wire connections
- ACC wire to the fuse
- Ground connection
- Controller box
4) Wiring.
Big circle – daylight running light
Next smaller circle – ground connector
Acc Squares – fused line
Box next to Acc – headlight
Right box – parking light
-      White wire = Headlight

· Blue wire = Parking light

· Black wire = Ground

· Red wire = ACC (with 15A fuse) line

· Yellow = 5 amp fuse
· Blue = 15 amp fuse
5-1)      Open Engine Fuse Box cover.
a. Remove 15A fuse shown.
5-2)     Put in ACC line and ground line.
a. Put ACC red fusedwire in same location as 15A fuse in step 5-1
b. Install ground wire in location shown.
Double check that both 15A, and 5A fuses are installed.
6 )   Locate the original Headlight / parking light wires in right hand side of the engine.
a. Low beam = blue /black (pos) wire connects to new blue wire using connector.
b. Headlight = red/purple(pos) wire connects to new white wire (positive) using connector.
