Idle Speed Adjustment

1. Perform “Engine Tune-up Preparation”.

2. Verify that the RPM PID is within the specification.

If not as specified, adjust the idle speed by

turning the AAS.

Specification

650—750 (70050) rpm

Caution

The TAS is set at the factory and must

not be adjusted. Any adjustment will

negatively effect the engine performance.

3. Connect the SST (System selector) to the DLC.

4. Set switch A to position 1.

5. Set the test switch to SELF TEST.

6. Press CLEAR to clear previously selected items.

7. Disconnect the SSTs.

Idle-up Speed Inspection

1. Perform “Engine Tune-up Preparation”.

2. Connect the SST (System selector) to the DLC.

3. Set switch A to position 1.

4. Set the test switch to SELF TEST.

5. Verify that the idle speed is normal. (See 01–10B–26 Idle Speed Adjustment.)

6. Verify that the RPM PID is within the specification.

If not as specified with all load conditions, inspect the IAC valve.

If not as specified with some load condition, inspect the related input switches, harnesses, and connectors.

Specification

*1 : Excludes temporary idle speed drop just after the electrical loads (E/L) are turned on.

*2 : Headlight is on, Fan switch is above 1st, Cooling fan is operating, Rear window defroster is on.

*3 : Steering wheel is fully turned.

*4 : A/C switch and fan switch are on.