                MPI Tuner and turbo module installation 

 #1. Install the MPI Tuner wire # 13 red to the # 71 PCM  [12 volt key on and run]

#2 Install the MPI Tuner wires, #1 black,  # 9 green and #6 blue  to the #24 PCM [ground]

#3Find the #88 wire at the PCM. This is the MAF signal wire. 

#4 Install the MPI Tuner wires,  #11 green/yellow, #10 black/white wire to the  #88 wire  

#5 Find and cut the # 60 O2 wire on the PCM.

#6 Install the #22 black/brown MPI wire to the engine side of the #60 wire.

#7 Install the #18 violet MPI wire to the PCM side of the #60 wire.

#8 Find the #89 PCM wire.  This is your TPS. Install the #7 brown MPI Tuner wire to the #89 PCM wire.

 #9 Find and cut the #26 red wire at the PCM. This is the coil driver signal wire for #1 and #4 cylinders. 

#10 Install the  MPI Tuner wire #8 yellow wire to the PCM side of #26 red. 

#11 Install the MPI Tuner wire #20 white to the long brown Turbo Module wire. Install the short brown to the engine side #26 red.

#12  Find and cut the #52 white wire at the PCM. This is the coil driver signal wire for#2 and # cylinders

#13 Install the MPI Tuner wire #5 white/red to the 

PCM side of the #52 white wire.

#14 Install the MPI Tuner wire #17 white blue to the long gray Turbo Module. Install the short gray Turbo Module wire to the engine side of the #52 white wire.

#15 Install the black Turbo  module wire to pin #51, PCM ground.

#16 install the red wire on the Turbo module to the red/white fuel pump feed wire. Please take your time to look at the wire real close. There is a white/red wire along side the red/white wire that we use.  If you make a mistake and use the white/red wire please call Mazda, you just fried your coils.  You will find it by following   the wire loom that enters under the front part of the center console. The wire loom will start right above the PCM loom and then run to the front center console under the vents/air duct.  This is the only 12 volt key on source that you can use for the blue wire in the Turbo module.  Do not use any other 12 volt key on source. 

[image: image1.png]MPI_TUNER - Global Parameter Setup by Modern Performance, Inc

Gobel| Ansog| Igion| Speed | Cyinders | Swich| Hodes  Sysem Corfig

Fosiive input pol
Posiive autput pol

Low level input

High requency

Low deviation

Interlaced signal

O missing tooth
Multcails

Nox ijection

Lanbda nput

Lanbda unlinear

Wide Dy

Oy loop enable

Oy loop reset on Deflect

el teloToNe ioteliote He Re TR0}

on
on
on
on
on
o
n
on
on
on
on
on
on
on

Clctete e loRetote OO Re ot

ot
ot
ot
ot
ot
ot
ot
ot
ot
ot
o
off
o
o

oK Cancel Apply

start| | | i America Online 9.0 Opt... | B] Mazda A instal. doc .. | [/} Adobe Acrabat - [Tech... | FE5 MPL_TUNER - 20 Maps ... [T MPL_TUNER - Global

=181]

[O®o @5 wioan




Thanks again

 Nick Mannarino

Phone 732-222-3679

e-mail mp23cc@aol.com 

